Fire Risk Assessment

It’s a legal requirement for you to conduct a fire risk assessment: -

- before you start operating in any premises

- where there are changes of use in any area of the building

- if there is an expected significant increase in people on site at any one time

- there are any management changes

The risk assessment is usually broken down into five elements: -

| 1. | Identify the hazards |
| --- | --- |
|  | This is usually broken into 3 areas: - - Sources of the ignition - Materials that are prone to burning - Flammable liquids Example Lighting; sockets; machinery; electrical equipment including computers; cooking equipment; Fire pits, BBQs; log burners Tee-pees; furniture, curtains and blinds, wood panelling; plastics; displays Cleaning products; fuel, BBQ lighters |
| 2 | Identify what / who is at risk |
|  | This will be anyone that comes into contact with your premises. When assessing the people/environment it’s important to work out where these people might be in relation to any fire; how many might be there at any one time and whether anyone might need special assistance Example Customers, staff, contractors, other visitors, neighbours, passers-by. |
| 3 | Take each hazard in turn and take measures to remove, reduce or put in place measures to protect people from the risk |
|  | It is almost impossible to remove fire risk entirely, but there are measures that can be taken to reduce the risk of a fire starting. For example: - With the potential sources of ignition - Ensuring that sockets are not overloaded - Having all electrical appliances Portable Appliance Tested (PAT) regularly - Prohibiting smoking on the premises - Replacing any open flame cooking equipment with electric With materials prone to burning including flammable liquids - Ensuring any materials used (including furniture) are fire retardant - Ensure that there are no combustible materials hanging over, being stacked against a potential source of ignition – for examples curtains over heaters or towels near boilers - Keeping flammable liquids well away from any potential source of ignition - Removing rubbish regularly With protecting people by: - Consider how you will know there is a fire (smoke detectors) - Considering how you will quickly alert everyone throughout the building if there is a fire (smoke alarms) - Considering which type of fire extinguishers you should have and where these should be placed throughout the building - Considering the quickest and safest exit point for people who may be located in different parts of the building - It is recommended that there are at least 2 escape routes from all parts of the building - Designing the premises so that there is easy access/wide corridors and passageways for people to pass along - Installing emergency lighting so that people can easily find their way out of the building (if yours is a small property, can you provide torches?) - Ensure that there is good, clear signage so that people know where to go in an emergency |
| 4 | Record, plan, inform, instruct and train |
|  | Even though it is only a legal requirement to record/document this information if you have 5 or more employees, it is good business practice for you to not only to keep a written record of your risk assessment but also to:- - Create an emergency plan that will detail the steps that will be taken and by whom in the event of a fire. - This plan should be displayed in staff and guest areas (for accommodation businesses, it needs to be in the guest bedrooms and in plain sight – often the inside of the door at eye level is a good place to ensure it is seen) - The plan should form part of every staff induction - Identify who is responsible for each element of a fire emergency including: - - Sounding the alarm - Using the fire extinguishers - Assisting any vulnerable people to leave the building - Liaising with the fire services - Keeping guests, staff informed - Ensure that all staff are properly trained to do what’s required of them and check their knowledge through regular fire drills and top up refresher training days |
| 5 | Review your plans |
|  | Review the fire risk assessment, emergency plan, training records and check all equipment periodically – logging each assessment and findings within a Fire Log Book. |

- Did you know that your local fire authority are good ports of call to check that you haven’t overlooked anything?

- There are also some great on-line resources that are venue/business specific:-

Outdoor attractions and events

Theatres, cinemas and music halls

Zoos, Farm Parks, Stables

Sleeping accommodation

Places where up to 300 people assemble

Places where more than 300 people assemble